Project: CC-OPTIR for MPs

Experiment: Bulk amino formaldehyde (AFR) MPs (aka fluorescent red MPs)

Data sources: mIRage OPTIR+Raman and LabRAM Raman

1. Sample collection details

a. Date of sampling: 20230804

b. Name of person(s) who sampled: Rebecca Parham

2. Data collection

a. Date of collection: 20230804

b. Name of collector: Rebecca Parham

c. Instrument: O-PTIR

i. 'core' file name: 20230804_1to5umAFR_bulk

ii. O-PTIR parameters

–Wavenumber: 1450 cm-1

–IR power: 20%

–Probe power: 5%

–Detector gain: 2x

–Averages: 10

iii. Raman parameters

–Resolution: 600 gr/mm

–Integration: 20s

–Averages: 6

iv. Comments

–Background spectra of aluminum-coated silicon was odd–the Raman spectra all had signs of fluorescence. There were no particles in the image where the spectra were taken, but the particles could have caused this issue. O-PTIR background spectra were fine.

–Two 8x8 mosaic images were taken with the 20x objective (one image "zoomed in" and the other "zoomed out").

–Largest peak in O-PTIR spectra of the particles was at ~1550 cm-1, and there were other small peaks in the range of 1320-1448 cm-1.

–Raman spectra of particles only exhibited fluorescence.

c. Instrument: HORIBA Raman

i. 'core' file name:

ii. Parameters

–Acquisition time:

–Accumulations:

–Delay time:

–Resolution:

–ND filter:

c. Instrument: HIROX

i. 'core' file name:

ii. Objective:

iii. Zoom:

d. Comments

Data analysis:

Include name, date, and any important details on analysis type, software used, and file names.